DATE HERE -  info, figures, whatever afterwards, DATE EVERYTHING

09/10/2022 – Image detection literature study

https://www.mygreatlearning.com/blog/introduction-to-edge-detection/

Edge detection described as: “a technique of image processing used to identify points in a digital image with discontinuities, simply to say, sharp changes in the image brightness”

There are several methods, all involving applying a filter matrix to the image at all possible points.

For all given pixels that the matrix will fit for, apply these coefficients for a given pixel, then sum the values and take this as the new value to be later normalized for the image:

This method results in the output being shrunk in dimensions, for an n*n image and a r*r filter the output image will be ( (n-r)+1)*( (n-r)+1).

Canny edge detection is the most common. The process involves:

- Convert the image to grayscale

- Reduce noise – as the edge detection that using derivatives is sensitive to noise, we reduce it.

- Calculate the gradient – helps identify the edge intensity and direction.

- Non-maximum suppression – to thin the edges of the image.

- Double threshold –  to identify the strong, weak and irrelevant pixels in the images.

- Hysteresis edge tracking – helps convert the weak pixels into strong ones only if they have a strong pixel around them.

Attempt to implement a Sobel filter in MATLAB:

https://www.geeksforgeeks.org/matlab-image-edge-detection-using-sobel-operator-from-scratch/

Advantages:

- Simple and time efficient computation (for small images, cannot go above 720p at least for Arduino)

- Very easy at searching for smooth edges (good for searching for edges of license plates)

Limitations:

- Diagonal direction points are not preserved always

- Sensitive to noise

- Not very accurate in edge detection

- Detect with thick and rough edges does not give appropriate results

Overall quality detection if the threshold values are set correctly:

Main concern here is the potential overhead and memory requirements needed for this level of image processing, alongside all the extra processing surrounding finding the bounding box of the plate.

10/10/2022 – Attempts at bounding edged image

Attempted to use code found online to create bounding boxes for the edge filtered image

https://uk.mathworks.com/matlabcentral/answers/35243-detecting-rectangle-shape-in-an-image

Not great, will need some tinkering with algorithm to detect correct bounding box.

Need to think of extra info that could help detect corners of plate. The corners themselves seem fairly distinct, perhaps a similar filter to the edge detection could be used to find the corners of the plate. A filter for this use may have been found by using the Robert Operator.

This could be utilized to filter out diagonal edges, which the licence plate has at its corners, take the highest intensity spots and use these to find the approximate location for the edges of the licence plate.

12/10/2022 – Checking image processing requirements

Looking at an image size calculator, the stated value of a 1280x720p image in 8bit monochrome would be around 0.85MB which is significantly more than most microcontroller SRAM.

https://toolstud.io/photo/filesize.php?imagewidth=1280&imageheight=720

Given that working with the actual pixels of the image itself will be necessary, it doesn’t seem feasible to save the image in a compressed format (unless some Realtime decompression can be performed). The image could potentially be streamed from the onboard storage if necessary, taking the data and reducing it to a very low-level form (4bit mono-chrome) for processing. Although the maximum level of SRAM I have found on an affordable board is 256KB of SRAM. I could find an external module to assist in storing images, but I am uncertain, even 4 bits per pixel on 480p leaves very little room for processing. I should probably work on getting the system working at a higher level before attempting to design anything though.

12/10/2022 – diagonal edge detection

The Robert filter doesn’t work well. I might need to design my own filter to get the corners I want or find another method.

15/10/2022 – Background reading

Some of the methods that ‘Computer vision for security applications’ talks about is the motion detection aspect. In early systems there where a large amount of false positives due to the system being unable to differ between camera shake and actual motion, along with rain and sunlight changes. The methods used to combat this were using low pass filtering to counter rain and a high pass filter to remove sunlight gradient effects and to make the image appear flatter. These filters could be used to get a range of motion frequency for a range of vehicle speeds to clean up the wake-up detection image.

‘An Automated Vehicle License Plate Recognition System’ uses a method of weighting the pixels of a RGB image differently to produce different styles of greyscale conversion. Looking at the MATLAB documentation for im2grey it also appears to use a weighted conversion to greyscale.

15/10/2022 – new idea for detecting bounds

The standard for the UK licence plate is such that, for any given plate, there will be 7 letters enclosed within a bounded box. An idea could be to find all areas that are bounded, such as with the code seen earlier in doc, and try to find a bounded area containing 7 bounded areas within it.

16/10/2022 – testing above idea

Tried to use far and angled image for testing a worse case scenario.

Still effective, checking and all letters appear to be creating bounded zones. Interesting effect is the fisheye lens that can be seen slightly on the image above. Given that I’m using relative pixel heights for this it could be an issue.

Checking the bounding data, some interesting effects on 1080p:

Both plates have clearly detectable bounded lettering, which creates 7 bounded areas. Tests at 720p also conducted:

Not great, pixel density is too great in some areas for plate letters to be counted as bounding areas. Seems like reducing the resolution reduces the number of pixels a fixed object size has to be represented in the image. Will try 480p with plate at closer range.

This 480p image is a closer range, hopefully better detectable.

Not much better, lettering still too pixel poor to create properly enclosed letter spaces. Notably the bounding box for the licence plate is still moderately accurate. In general I think I should aim for the highest resolution feasible when working with hardware.

Possibility of using the bwareafilt function to get all white pixel shapes that have a size within a given range:

The above is a 1080p image with filters set to take all shapes within 100-1000 pixels, not working for detection by itself but could be useful elsewhere. Problems would probably emerge as tracked plate will change pixel size across image sequence.